Modem Diagnostic Report

Machine: N096AV  |  Modem: Sierra Wireless EM7565

Date: June 18, 2026  |  Version: 2.0 (Updated after firmware upgrade)

Prepared by: Field Diagnostics

# 1. Executive Summary

Machine N096AV has no cellular connectivity. After extensive diagnosis including a successful firmware upgrade from SWI9X50C_01.08.04.00 to SWI9X50C_01.14.02.00, the modem continues to report NO IMSI internally despite the SIM card being physically readable and signal being strong (CSQ 25/31). All software-level recovery options have been exhausted. The modem hardware is defective and must be replaced.

# 2. System Environment

| Component | Details |
| --- | --- |
| Machine ID | N096AV |
| OS | Ubuntu 20.04 LTS |
| Modem | Sierra Wireless EM7565 (Qualcomm Snapdragon X16 LTE-A) |
| Modem Serial | UF22173591021052 |
| Original Firmware | SWI9X50C_01.08.04.00 (August 2018) |
| Updated Firmware | SWI9X50C_01.14.02.00 (August 2020) |
| USB Mode | QMI (qmi_wwan) |
| SIM Provider | Movistar Colombia (MCC/MNC: 732123) |
| SIM IMSI | 732123667007142 |
| SIM ICCID | 8957123412113518379F |
| Location | Same physical location as working machines N133AV and N151AV |

# 3. Initial Symptom

The PPP connection established LCP and PAP authentication successfully but timed out waiting for IPCP (IP address assignment). The ppp0 interface showed TX bytes growing but RX bytes frozen at 62, confirming the carrier accepted the PPP handshake but refused to assign an IP address.

PAP authentication succeeded

IPCP: timeout sending Config-Requests

Connection terminated. Modem hangup

# 4. Diagnostic Steps Performed

## Step 1 — ModemManager Disabled State

ModemManager detected the modem via QMI (/dev/cdc-wdm0) but reported it as disabled at boot. The modem was not enabled before pppd dialed, causing every PPP session to fail at IPCP. This was fixed by deploying the modem-enable.service systemd unit (same fix applied fleet-wide to N133AV and N151AV).

## Step 2 — ttyUSB Port Mapping Mismatch

The udev symlinks were misaligned due to FTDI device enumeration order at boot:

| Symlink | Points To | Actual Function |
| --- | --- | --- |
| modemAT | ttyUSB4 | GPS port — AT commands not accepted |
| modemPPP | ttyUSB5 | Actual AT interface |

## Step 3 — Operator Registration Drift

After enabling the modem, it attempted to register on operator 732360 (ETB Colombia) instead of 732123 (Movistar). Manual registration timed out. Band lock was attempted targeting Movistar Colombia LTE Band 4 (1700MHz) and Band 28 (700MHz) via AT!BAND command, but registration remained idle.

## Step 4 — QMI UIM Stack Failures

ModemManager repeatedly failed to initialize the modem:

QMI operation failed: Illegal SIM/USIM application state

state changed (locked -> failed)

failed reason: sim-missing

Despite these errors, the SIM remained physically readable via AT commands (AT+CIMI returned valid IMSI). The QMI and AT stacks were permanently out of sync.

## Step 5 — Recovery Attempts (Pre-Firmware Upgrade)

All of the following recovery methods were attempted without success:

- AT+CFUN=0 / AT+CFUN=1 — SIM power cycle. IMSI remained unloaded in modem stack.

- AT!RESET — Full modem reset. Temporarily cleared failed QMI state but NO IMSI persisted.

- mmcli -m 0 --reset — ModemManager reset. Same result.

- AT!BAND=00 — Reset to all bands. No change in registration.

- mmcli --set-allowed-modes=4g / 3g — Mode forcing. Registration stayed idle.

- mmcli --3gpp-register-in-operator=732123 — Manual Movistar registration. Timeout every time.

## Step 6 — Firmware Upgrade

A firmware upgrade was performed using qmi-firmware-update (libqmi-utils 1.30.4) since the proprietary fwdwl-litehostx86_64 tool failed repeatedly with error 100 (Firehose protocol timeout).

qmi-firmware-update -u -w /dev/cdc-wdm0 --device-open-qmi --ignore-mm-runtime-check \

SWI9X50C_01.14.02.00.cwe SWI9X50C_01.14.02.00_GENERIC_002.035_003.nvu

The firmware upgrade completed successfully:

original firmware: SWI9X50C_01.08.04.00 (2018)

new firmware:      SWI9X50C_01.14.02.00 (2020)

firmware update operation finished successfully

## Step 7 — Post-Firmware Assessment

After the firmware upgrade, the modem was re-evaluated. The NO IMSI condition persisted unchanged:

| Parameter | Value | Assessment |
| --- | --- | --- |
| Firmware | 01.14.02.00 | Successfully upgraded |
| AT+CSQ (signal) | 25,99 (-83dBm) | Good signal |
| AT+CIMI (SIM IMSI) | 732123667007142 | SIM readable via AT |
| !GSTATUS MM state | NO IMSI | Modem stack not loading SIM |
| !GSTATUS PS state | Not attached | Cannot attach to network |
| Registration | idle | Never registers |
| Network scan | Timeout | Cannot scan operators |

# 5. Key Evidence for Hardware Fault

The following combination of facts conclusively points to a hardware defect in the modem's SIM interface:

- Signal strength is good (CSQ 25/31, WCDMA RSSI -64dBm) — coverage is not the issue.

- All other machines at the same physical location connect successfully to Movistar.

- SIM card is valid and readable via AT+CIMI — the SIM itself is not defective.

- NO IMSI persists across two different firmware versions (01.08.04.00 and 01.14.02.00).

- NO IMSI persists across multiple modem resets (AT!RESET, AT+CFUN cycles, mmcli --reset).

- QMI UIM stack consistently reports 'Illegal SIM/USIM application state' — a hardware-level failure in the QMI SIM interface controller.

- AT stack and QMI stack are permanently out of sync — a known symptom of failed SIM interface hardware.

- All software recovery options exhausted without resolution.

# 6. Note on Firmware Tool Failures

The proprietary Sierra Wireless firmware tool (fwdwl-litehostx86_64) failed consistently with error 100 (litefw_DownloadFW failed: 100) and error 103 (MBIM open response type error). The root cause was failure to complete the Firehose protocol handshake (sahara message timeout). The firmware was ultimately upgraded successfully using the open-source qmi-firmware-update tool from libqmi-utils.

# 7. Conclusion

The Sierra Wireless EM7565 modem on machine N096AV has a permanent hardware defect in its SIM interface controller. The defect manifests as the modem's internal QMI/UIM stack being unable to load the IMSI from the SIM card, preventing any network registration. This fault persists across firmware versions, resets, and all known software recovery procedures. The modem must be physically replaced.

# 8. Recommendations

| Priority | Action | Details |
| --- | --- | --- |
| Immediate | Replace modem | Replace the Sierra Wireless EM7565 on N096AV. The current modem is not recoverable. |
| Before deployment | Verify QMI mode | After replacement confirm usb-devices shows qmi_wwan driver and mmcli -L detects the modem. |
| Before deployment | Deploy modem-enable service | Install modem-enable.service to ensure ModemManager enables the modem at boot before pppd dials. |
| Before deployment | Upgrade firmware | Flash new modem to SWI9X50C_01.14.02.00 using qmi-firmware-update before deployment. |
| Fleet-wide | Firmware audit | Check all machines for SWI9X50C_01.08.04.00 firmware. Upgrade any found using qmi-firmware-update. |