Modem Diagnostic Report

Machine: N120AV  |  Modem: Sierra Wireless EM7565

Date: June 11, 2026  |  Prepared by: Field Diagnostics

# 1. Executive Summary

Machine N120AV experienced a complete cellular connectivity failure. After systematic diagnosis, the root cause was determined to be a damaged Sierra Wireless EM7565 modem. The modem is unable to detect the SIM card, preventing network registration and PPP data session establishment. A modem replacement is required.

# 2. System Environment

| Component | Details |
| --- | --- |
| Machine ID | N120AV |
| OS | Ubuntu 20.04 LTS |
| Modem | Sierra Wireless EM7565 (Qualcomm Snapdragon X16 LTE-A) |
| Modem Serial | UF21143469021050 |
| SIM Provider | Movistar Colombia (MCC/MNC: 732123) |
| Connectivity | PPP via pppd + APN: internet.movistar.com.co |
| Remote Access | OpenVPN over PPP |

# 3. Initial Symptom

The PPP connection established (LCP and PAP authentication succeeded) but no IP address was assigned, causing the connection to terminate with IPCP timeout:

PAP authentication succeeded

IPCP: timeout sending Config-Requests

Connection terminated.

The interface ppp0 showed heavy TX traffic but near-zero RX bytes, confirming outbound packets were sent but no responses were received from the carrier.

# 4. Diagnostic Steps Performed

## Step 1 — PPP Traffic Analysis

Checked interface statistics on ppp0:

RX: bytes=62   packets=5

TX: bytes=145399   packets=2194

Confirmed: the modem was transmitting but receiving nothing — indicating a dead data session, not a routing or DNS issue.

## Step 2 — ModemManager vs pppd Conflict Check

Verified ModemManager status on the working reference machine (N133AV) vs N120AV. On N133AV, ModemManager successfully managed the modem via QMI (/dev/cdc-wdm0) with qmi-proxy running. On N120AV, ModemManager failed to initialize the modem entirely.

## Step 3 — USB Composition Mode Identified

Ran usb-devices to inspect the modem's USB interface configuration:

| Interface | N120AV (Broken) | N133AV (Working) | Impact |
| --- | --- | --- | --- |
| If#0 | cdc_mbim (MBIM) | qmi_wwan (QMI) | Wrong protocol mode |
| If#2 | qcserial (AT) | qcserial (AT) | OK |
| If#3 | qcserial (PPP) | qcserial (PPP) | OK |

The modem was in MBIM composition mode instead of QMI mode, preventing ModemManager from initializing it correctly.

## Step 4 — Attempted USB Composition Switch

Attempted to switch the modem from MBIM to QMI mode using AT commands via minicom on /dev/modemAT:

AT!ENTERCND="A710"  ->  OK

AT!USBCOMP?         ->  0000100D (diag,nmea,modem,mbim)

AT!USBCOMP=1,1,0000500D  ->  OK

AT!RESET

After reset the modem came back still in MBIM mode — the composition change did not persist.

## Step 5 — Interrupted Firmware Upgrade

A firmware upgrade was attempted using Sierra Wireless tools but was interrupted mid-process with Ctrl+C. After reboot, the following symptoms appeared:

- AT+CIMI returned ERROR (cannot read SIM IMSI)

- AT+CSQ returned ERROR (cannot read signal strength)

- AT+CREG? returned ERROR (cannot access network registration)

- AT!GSTATUS? showed: MM (CS) state: NO IMSI, PS state: Not attached

## Step 6 — Post-Reimage Assessment

A full OS reimage was performed on N120AV. After reimaging, the modem state was re-evaluated:

- Modem still in MBIM mode (USB composition stored in modem NV memory, not OS)

- AT command ports (ttyUSB3, ttyUSB4) returned no response

- ModemManager: No modems were found

## Step 7 — MBIM Interface Communication

Installed libmbim-utils and communicated with the modem via its MBIM interface (/dev/cdc-wdm0):

| MBIM Query | Result |
| --- | --- |
| --query-device-caps | Firmware: SWI9X50C_01.14.02.00 (upgraded successfully) |
| --query-subscriber-ready-status | Ready state: sim-not-inserted |
| --query-signal-state | RSSI: 99 (no signal) |
| --query-registration-state | Register state: searching (no SIM to register) |
| --set-radio-state=off/on | No change — SIM still not detected after radio cycle |

# 5. Conclusion

The diagnostic evidence points conclusively to a damaged Sierra Wireless EM7565 modem. The sequence of findings that led to this conclusion:

- The firmware upgrade completed successfully (firmware version confirmed as SWI9X50C_01.14.02.00 via MBIM).

- The SIM card itself is functional — when tested in other machines it operates correctly.

- The modem's SIM interface is non-functional: sim-not-inserted is reported even with a known-good SIM physically present.

- AT command ports are completely unresponsive despite no process holding them.

- Radio power cycling via MBIM did not restore SIM detection.

- OS reimage had no effect — the fault is in the modem hardware/NV memory, not the operating system.

The most likely cause is that the interrupted firmware upgrade corrupted the modem's NV (non-volatile) memory, permanently damaging the SIM interface controller. This type of damage is not recoverable through software means.

# 6. Recommendation

| Priority | Action |
| --- | --- |
| Immediate | Replace the Sierra Wireless EM7565 modem on N120AV |
| Before replacement | Verify replacement modem enumerates in QMI mode (not MBIM) after installation |
| Fleet-wide | Audit all machines for USB composition mode using: usb-devices ¦ grep -A10 sierra |
| Preventive | Never interrupt a Sierra Wireless firmware upgrade — if it must be stopped, allow it to complete or perform a full recovery flash |

# 7. Reference — Working vs Broken Modem State

| Parameter | N133AV (Working) | N120AV (Damaged) |
| --- | --- | --- |
| USB Mode | QMI (qmi_wwan) | MBIM (cdc_mbim) |
| ModemManager | EM7565 detected | No modems found |
| AT Commands | Responsive | No response |
| SIM Detection | IMSI readable | sim-not-inserted |
| Network Reg. | Registered (Movistar) | Searching / Failed |
| PPP Result | Connected, IP assigned | IPCP timeout |
| Firmware | SWI9X50C_01.08.04.00 | SWI9X50C_01.14.02.00 |